/*!

\page import_export_geom_obj_page Importing/exporting geometrical objects

In Geometry module you can import and export geometrical objects

from/into BREP, IGES, STEP files. The mechanisms of import and export

are implemented via plug-ins, which gives the opportunity to

expand the range of available formats by adding more plug-ins (for

example, CATIA 5).

<em>To import geometrical objects from a BREP, IGES, STEP file:</em>

\par

From the \b File menu choose \b Import. In the opening dialog box \b Import

select the required format of the file for importation and search for

a *.brep, *.iges or *.step  file.

\image html geomimport.png

\par

Select the required file and click \b Open. Your file will be imported in

the module and its contents (geometrical object) will be displayed in

the <b>Object Browser</b>.

\note If the selected file is in IGES format and the length is not

expressed in meters, it will be suggested to scale the model into the

metric system (see the picture below). This feature can be helpful if

some wrong units have been written to the IGES file by a

3rd-party software.

\image html iges_unit.png

\par

\note If the selected file is in ACIS format (the use of this format

requires licensing) and the file contains names for some shapes, it

will be suggested to create groups by types (solid, face, edge and

vertex) for all named shapes:

\image html sat_named_shapes.png

\par

\note It is possible to re-open from the initial file a previously imported shape if

the file has been changed on disk. For this, select

<b>Reload From Disk</b> in the context menu of the imported

shape. The reloaded shape will have the same representation parameters

as before this operation.

\image html geomimport_reopen.png

\n <em>To export geometrical objects into a BREP, IGES, STEP

file:</em>

\par

Select the object you wish to export, then from the \b File menu choose

\b Export. In the opening dialog box \b Export define the required format,

the name and the location of the file for exportation.

\image html geomexport.png

\par

Click \b Save to confirm your exportation.

Our <b>TUI Scripts</b> provide you with useful examples of the use of

\ref tui_import_export_page

*/